# SAUTER viaSens®181: Air quality sensor (VOC)

## Area of application

Measurement of the indoor air quality for energy-efficient controlling of the room climate.

## Performance features

- Sensor for measuring the room air quality for ventilation according to needs (for example, tobacco smoke, kitchen fumes, human body odours, or gases given off by materials such as furniture, carpets, adhesives, etc.)

- Sensor for controlling the room climate in combination with room automation systems

- Indoor air quality measurement; VOC contamination based on VDMA 24772

- Fast response time and high precision

- Calibrated ex works and ready to use immediately

- Automatic, continuous drift compensation against ageing and seasonal effects

- Low energy requirement of the ventilation system during the warming up time of the sensor

- Quick, easy overview of the indoor air quality via a multi-coloured LED

- Parameterisation with the CASE Sensors software

- Supply voltage and output signal galvanically isolated

- Room sensor in a wide range of designs and colours

- Device insert fits into frame with 55 x 55 mm aperture

## Technical data

General information

| Power supply (SELV): | 24 V~, 24 V= |
| --- | --- |
| Analogue output: | 0...10 V |
| Connection terminals: | Pluggable screw terminal for conductor cross-section of max. 1.5 mm2 |
| Dimensions, W x H x D: | 59.7 x 59.7 x 53 mm |
| VOC (volatile organic compounds) |  |
| Measuring range: | 0...100 %IAQ |
| Accuracy: | ± 20 % |
| Time constant: | < 60 s |
| Warming-up time: | 30 min. |

Accessories

| 0940240***: | Frames, mounting plates and adaptors for third-party frames and surface mounting |
| --- | --- |
| 0300360001 | CASE Sensors USB connection set, including software |

Manufacturer:			SAUTER

Make:				EGQ181SF203